# Falco Infinity / Minotaur V13
Reverse-Engineering Notes

Memory map, I/O map, banking logic, video memory organization, audio control, evidence, and remaining work

Status. This note records the state of the reverse engineering after continuity tracing of the two-layer motherboard and static analysis of both known ROM sets. Findings are explicitly separated into observed facts, firmware-supported conclusions, and hypotheses where useful.

| Confidence | Meaning |
| --- | --- |
| Observed | Directly traced on the PCB or read from component/board markings. |
| Firmware-confirmed | Behavior follows directly from executable ROM code, memory tests, or embedded strings. |
| Strong inference | Multiple independent observations agree, but the exact circuit/equation has not yet been measured. |
| Open | Not yet determined, or several plausible implementations remain. |

# 1. Machine and board identification

Terminal: Falco Infinity. The installed custom ROM labels further identify the firmware target as “Infinity MP-G”, consistent with a multi-personality graphics terminal.

Motherboard assembly: 230300-113 Rev D.

Bare PCB markings: PWB MINOTAUR V13; 130165-013; FALCO DATA PRODUCTS; COPYRIGHT 2/89.

CPU: Z80-family processor, with Z80 SIO and Z80 CTC peripherals.

PCB construction: Two copper layers. This makes continuity tracing unusually useful; hidden vias or inner-layer routing are not a concern.

## ROM sets

| Set | Physical label / provenance | Embedded firmware ID | Notes |
| --- | --- | --- | --- |
| GC Services | “GC Services / Infinity MP-G / MP 921104 / SR0, SR1” | (c) 1991 FDP, Inc O731 | Installed in terminal. “921104” plausibly denotes 1992-11-04, but that interpretation is not proven. |
| Loose Falco set | Original-looking Falco labels; preprinted copyright 1984; stamped 189733 | (c) 1992 FDP, Inc O733 | O733 is newer than O731. The 1984 label date is therefore not the firmware build date. The ending “733” in 189733 is likely related to O733, but the numbering scheme is not known. |

Both sets are two 64 KiB ROM images. They are ordinary byte-wide Z80 code/data, not even/odd interleaved images. The firmware contains readable setup strings, terminal personality names, diagnostics, and executable code at the addresses implied by the map below.

# 2. CPU memory map

| CPU address | Function | Implementation / evidence | Confidence |
| --- | --- | --- | --- |
| 0000-7FFF | Fixed 32 KiB program ROM | ROM0 upper 32 KiB. Z80 A15 is inverted (and then buffered) to ROM0 A15, so CPU A15=0 selects physical ROM0 A15=1. Reset/boot code in the ROM contents is consistent with this mapping. | Observed + firmware-confirmed |
| 8000-BFFF | 16 KiB banked program ROM window | Bank number is written through I/O block 00-0F. The PAL receives Z80 D0-D2, enough for an 0-7 bank value. Firmware uses multiple banks; the two 64 KiB ROMs provide 96 KiB beyond the fixed 32 KiB, exactly six 16 KiB banks. | Firmware-confirmed; exact PAL equations inferred |
| C000-DFFF | 8 KiB CPU data RAM, battery-backed | 6264-class 8 KiB SRAM. Directly on the Z80 address bus; one diode drop from the battery positive terminal. Firmware deliberately tests exactly this region and keeps its stack/globals here. | Observed + firmware-confirmed |
| E000-FFFF | 8 KiB video-memory aperture | PAL selects the video ASIC for this region. Firmware changes video register 2 (port 52h) and accesses E000-FFFF to select/test character RAM and screen-RAM pages. | Observed + firmware-confirmed |

## Physical ROM organization

ROM0 physical 8000-FFFF  -> CPU 0000-7FFF  (fixed 32 KiB)
ROM0 physical 0000-3FFF  -> CPU 8000-BFFF  (bank 0)
ROM0 physical 4000-7FFF  -> CPU 8000-BFFF  (bank 1)
ROM1 physical 0000-3FFF  -> CPU 8000-BFFF  (bank 2)
ROM1 physical 4000-7FFF  -> CPU 8000-BFFF  (bank 3)
ROM1 physical 8000-BFFF  -> CPU 8000-BFFF  (bank 4)
ROM1 physical C000-FFFF  -> CPU 8000-BFFF  (bank 5)

The six-bank ordering above is the natural arrangement implied by the ROM contents and hardware. Banks 0-5 account for every byte not used by the fixed image. Whether bank values 6 and 7 are disabled, aliases, or otherwise decoded remains open.

# 3. Top-level I/O map

A 74x138 performs the coarse I/O decode. Its select inputs are A=Z80 A4, B=A5, C=A6. The enables are /G2A=A7, /G2B=/IORQ, and G1=/M1. Thus it is enabled for ordinary I/O cycles in the lower half of the 8-bit port space and disabled during Z80 interrupt acknowledge.

| Ports | 74x138 output | Destination | Interpretation |
| --- | --- | --- | --- |
| 00-0F | Y0 | PAL16L8ACN pin 1 | ROM-bank / CPU memory-decode programming block. Firmware normally uses port 00h. |
| 10-1F | Y1 | Unpopulated expansion connector/interface | Expansion I/O block; function unknown. |
| 20-2F | Y2 | No connection found | Apparently unused. |
| 30-3F | Y3 | No connection found | Apparently unused. |
| 40-4F | Y4 | 74HCT374 clock | Write-only 8-bit audio/buzzer control latch; firmware normally uses port 40h. |
| 50-5F | Y5 | Video ASIC | Sixteen directly addressed video registers; Z80 A0-A3 reach the ASIC. |
| 60-6F | Y6 | Z80 SIO chip select | Serial I/O register block. |
| 70-7F | Y7 | Z80 CTC chip select | Counter/timer register block; channel 2 participates in buzzer/tone generation. |

# 4. PAL16L8ACN: ROM banking and CPU memory decode

The PAL is the central CPU-side memory glue. It is a PAL16L8ACN: a nominally combinational device, but its bidirectional output macrocells have feedback into the logic array and can therefore be used to form level-sensitive internal state. That is a good fit for the observed need to remember the three bank bits written at port 00h.

| PAL pin | Observed destination / source | Likely role |
| --- | --- | --- |
| 1 | 74x138 Y0 | Port 00-0F write/select input. |
| inputs (3) | Z80 D0, D1, D2 | Three-bit ROM bank value. |
| 12 | ROM0 /E | ROM0 chip enable / fixed+banked ROM decode. |
| 13 | ROM1 /E | ROM1 chip enable / bank decode. |
| 14 | ROM0 A14 and ROM1 A14 | Address mux: CPU A14 in fixed region versus bank-state bit in the banked window. |
| 15 | ROM1 A15 | Upper bank-address control; may also participate in internal feedback/state. |
| 16 | No external trace found | Strong candidate for internal-only feedback/state node. |
| 17 | No external trace found | Strong candidate for internal-only feedback/state node. |
| 18 | 6264 data-RAM /CE1 | Decode for C000-DFFF battery-backed RAM. |
| 19 | Video ASIC | Decode/select for E000-FFFF video-memory aperture. |

ROM0 A15 is not produced by the PAL: it is simply Z80 A15 passed through an inverter and then a buffer. This directly explains why ROM0 upper 32 KiB is fixed at CPU 0000-7FFF while ROM0 lower 32 KiB supplies the first two 16 KiB switchable banks.

Pins 16 and 17 appear to have no routes leaving the DIP footprint on this two-layer board. That is compatible with using them only as internal PAL feedback nodes. This is a strong hypothesis, not yet a recovered fuse equation.

# 5. Video-memory organization

The video subsystem has at least two physically distinct SRAM address buses. Four 32 KiB SRAM positions near the video ASIC share their own address bus; two sockets are populated and two are empty. A separate 8 KiB SRAM near the ASIC has a different A0 and appears to be a separate character-generator memory. Neither arrangement looks like ordinary SRAM placed directly on the Z80 address bus.

| Resource | Physical memory | CPU access | Evidence / interpretation |
| --- | --- | --- | --- |
| Screen memory | 2 x 32 KiB populated + 2 x 32 KiB empty sockets = 64 KiB installed, 128 KiB possible | 8 KiB pages through E000-FFFF; page selection is controlled through video register 2 at port 52h | Firmware probes page aliasing, derives a 2/4/8/16-page configuration, and tests selected 8 KiB pages. The observed 64/128 KiB hardware exactly matches 8/16 pages. |
| Character-generator RAM | Separate 8 KiB SRAM beside video ASIC | E000-FFFF when video register 2 is programmed for character RAM | Firmware writes 08h to port 52h, tests E000-FFFF, and reports failure as “Char Gen”. |
| CPU data/NVRAM | Separate 8 KiB 6264 near Z80/SIO | Direct C000-DFFF | Not part of video buses; battery backed and explicitly tested as “Data RAM”. |

## Working architectural model

Z80 CPU bus
              +---------------+----------------+
              |               |                |
          program ROM     8 KiB BBRAM       SIO / CTC
          via PAL          C000-DFFF         I/O devices
              |
          bank port 00h

Z80 CPU bus ------------------------------+
                                           |
                                   +-------v-------+
ports 50h-5Fh, A0-A3 ------------>|  Video ASIC   |
                                   |               |
E000-FFFF aperture -------------->| CPU/video RAM |
                                   | arbitration   |
                                   +---+-------+---+
                                       |       |
                              private bus     private bus
                                       |       |
                                64/128 KiB   8 KiB
                                screen RAM   char RAM

The exact arbitration mechanism and video-side addressing are not yet known, but the physical bus separation and firmware-visible 8 KiB aperture make this architecture the simplest fit to the evidence.

# 6. Port 40h: buzzer/audio control

The 74HCT374 initially looked like a bank latch, but tracing and firmware analysis identify it much more convincingly as part of the sound circuit.

Observed wiring: Z80 D0-D7 feed the 74HCT374 data inputs. 74x138 Y4 clocks the latch. Its outputs run into a cluster of unequal-value resistors immediately adjacent to the buzzer.

Firmware behavior: The terminal keeps a software shadow of the port-40h value in battery/data RAM. The low six bits and upper two bits are manipulated separately. Firmware sometimes writes the full byte for a finite interval and then writes only value & C0h, clearing the low six outputs while preserving the upper two.

CTC correlation: Code that uses the upper two bits also programs CTC channel 2 (port 72h), strongly linking the latch to tone generation rather than to unrelated system control.

Working interpretation: The resistor network forms one or more crude DAC/level networks for the passive buzzer. The low six bits are likely a fine-grained short-duration level such as key-click strength; the upper two bits are likely a coarser sustained bell/tone level gated or timed by CTC channel 2. Exact bit-to-function assignment remains to be measured.

The direct active-low Y4 output feeding the positive-edge-triggered 74HCT374 clock is also sensible: the latch captures the data bus on the rising edge at the end of an I/O cycle. Unless additional qualification exists, all 40h-4Fh addresses alias the same write-only latch.

# 7. Firmware discoveries beyond the maps

Terminal personalities: The ROM contains an emulation/personality table including FALCO, VT100, VT300, VT52, TVI955, TVI950, WYSE60, WYSE50, TVI910, TVI920, TVI925, HZ1500, ADDSVP, DG200, DG210, 3101-1, and 3101-2.

Diagnostics: Embedded diagnostics explicitly name ROM1, ROM2, Screen, Data, and Char Gen, which helped tie firmware tests to physical memories.

Version relationship: O731 and O733 are closely related builds from the same firmware family, not unrelated programs. Large code regions match while routines/data move as the software evolves.

Setup/NVRAM: The battery-backed C000-DFFF region almost certainly stores terminal setup/configuration in addition to ordinary working RAM. The firmware can be used to locate the persistent structures.

Hardware consolidation: Compared with the older Falco 500-series schematic, this 1989 Infinity board appears to have moved much of the CPU decode/banking into the PAL and much of the video timing/address generation/arbitration into the large PLCC ASIC, explaining the sharply reduced TTL count.

# 8. What remains unknown, and the best way to determine it

| Question | Current status | Best next method |
| --- | --- | --- |
| Exact video ASIC register map (50h-5Fh) | Only register 2 / port 52h is partly understood; it controls CPU access to screen/character memory and page selection. | Static software analysis first: enumerate every IN/OUT to 50h-5Fh in both ROM revisions, group call sites by nearby strings/functions, and identify initialization sequences. Then validate on hardware with a logic analyzer while changing one setup option at a time. |
| Exact screen-memory layout | Capacity/page count is understood; byte organization, text/attribute/graphics layout, and video fetch sequencing are not. | Trace SRAM /CE, /OE, /WE, A13/A14 and data paths; correlate with firmware screen-clear/scroll routines. Capture ASIC-to-SRAM activity during simple static test screens. |
| Exact character-generator RAM format | 8 KiB device and CPU aperture selection are understood; glyph dimensions/packing and download behavior are not. | Find firmware routines that write E000-FFFF with port 52h=08h; correlate write counts/strides with displayed characters. A logic-analyzer capture of a known glyph update would settle addressing quickly. |
| PAL16L8 equations and bank 6/7 behavior | External outputs and major function are understood; exact feedback equations/state encoding are not. | Best: read the PAL fuse map if it is unsecured and a compatible programmer is available. Otherwise exercise OUT 00h with bank values 0-7 while observing ROM /E, A14 and ROM1 A15; a tiny truth table would recover the externally relevant equations. |
| Exact port-40h audio bit meanings | Audio role is strong; exact resistor weights and key-click/bell allocation remain inferred. | Trace each 374 Q output through the resistor network and CTC ZC/TO2 to the buzzer. Then scope the buzzer while changing key-click/bell setup values or patching the shadow byte. |
| Expansion block 10h-1Fh | Y1 reaches an unpopulated expansion interface, function unknown. | Trace connector pins to nearby logic and search firmware for IN/OUT accesses in 10h-1Fh. Check whether O731/O733 differ in expansion support. |
| SIO/CTC board-level routing | I/O blocks are known but channel clocks, modem/control lines, and connector destinations are not fully mapped. | Continuity trace peripheral pins to connectors/oscillators. Use standard Z80 SIO/CTC register semantics to annotate firmware initialization. |
| GC Services customization | Custom labels are known; exact behavioral differences from stock O733 are not. | Binary diff at function level: map corresponding routines/strings, identify code unique to O731/GC build, and inspect setup defaults/protocol handling rather than relying on raw byte offsets. |
| Battery-backed configuration layout | Persistent RAM is identified but record format is unknown. | Dump NVRAM, change one terminal setting, dump again, and diff. Repeat for a few orthogonal settings; then match locations to firmware references in C000-DFFF. |

## Recommended order of effort

1. Video-register analysis in software. Highest return: the ROM is effectively the driver documentation for the proprietary ASIC, and no risky physical probing is required.

2. Finish the audio circuit. A few continuity measurements plus an oscilloscope capture should completely document port 40h and CTC channel 2.

3. Recover the PAL truth table. If the fuse map cannot be read, testing bank values 0-7 at the ROM pins should still resolve every externally significant equation.

4. Decode NVRAM/configuration. Simple before/after dumps should expose persistent setup structures and may also reveal model/customer defaults.

5. Only then probe the video ASIC pins. Use physical ASIC tracing mainly to answer questions the software interface cannot resolve: bus arbitration, video fetch timing, external clocking, and SRAM pin roles.

# 9. ROM image checksums

SHA-256 values for the four uploaded dumps, useful for archive metadata and later comparison:

| File | Size | SHA-256 |
| --- | --- | --- |
| falco-gc-services-000.rom | 65536 | c9c69d23b20f4f85881ba2c9dab53024ecf6297154fd86c14e4ec46c4c1fa909 |
| falco-gc-services-001.rom | 65536 | 347094df711cc42649787c06b87018fce9270ac5672e3cbbbd6920911a38ab8e |
| falco-original-000.rom | 65536 | 9049a50e9c6766eb44b98362543556db61c01e3ca07d845c06f4e1a31d29df2e |
| falco-original-001.rom | 65536 | 6e66ef5f345d33eb513e95462b2ff69b7bbe6d33eaffd4dbc1d909a65c7112fb |

# 10. Compact reference

CPU MEMORY
0000-7FFF   fixed ROM (ROM0 upper 32 KiB)
8000-BFFF   16 KiB banked ROM window; bank via port 00h / PAL D0-D2
C000-DFFF   8 KiB battery-backed CPU data RAM
E000-FFFF   8 KiB video-memory aperture

I/O
00-0F       PAL / ROM bank and memory glue (normally 00h)
10-1F       unpopulated expansion
20-2F       unused?
30-3F       unused?
40-4F       74HCT374 audio/buzzer level latch (normally 40h)
50-5F       video ASIC, 16 registers (52h = memory-page/access control)
60-6F       Z80 SIO
70-7F       Z80 CTC

VIDEO MEMORY
Screen RAM  64 KiB installed; sockets support 128 KiB; CPU sees 8 KiB pages
Char RAM    separate 8 KiB; CPU maps it at E000-FFFF via video register 2

KEY GLUE
PAL16L8ACN  ROM enables, ROM bank addressing, C000-DFFF RAM select,
             E000-FFFF video select, probable feedback-held bank state
74x138      top-level lower-half I/O decode
74HCT374    write-only 8-bit audio/buzzer control latch